Speak Freely for Unix

(formerly netfone)

Development Log

26 August 1995

John Gilmore (gnu@toad.com) encountered an undefined symbol irint when

building on SunOS 4.1.3_U1.  Apparently this function, which is part

of the main C library in 4.1.1, was moved somewhere else in 4.1.3.  To

put an end of this irint() nonsense once and for all, I rewrote the

expression in soundbyte.c to eliminate the call to irint() entirely.

John also suggested making the distribution tar extract into a

netfone-<version> directory as most GNU packages do.  Starting with

release 5.1, which will extract into a netfone-5.1 directory, that's

how it'll be.

29 August 1995

Added logic to mike to calculate the precise number of samples which

will fit in a sound buffer to optimally fill a single UDP packet.

John Gilmore pointed out that sending lots of individual packets on

our own is actually more efficient than forcing the network driver to

break large packets into fragments.  It also reduces the sound delay,

since we don't have to wait for a large sound buffer to be filled

before getting it on it's way to the other end.

Note that even though mike will now never send a packet larger than

512 bytes, speaker continues to be able to handle packets with as

many as 8000 samples and thus remains compatible with older versions of

mike.

31 August 1995

Added -d and -q switches to speaker which, if specified, override the

debug mode specified in incoming packets.  The -d switch is

particularly handy when you want to log every packet and don't have

control over the sending end.  The -q switch allows running speaker in

the background without worrying about where the output is going to go

is somebody starts sending with debug mode on.

To reduce network traffic and improve conference call performance,

I made push-to-talk (-b) mode the default in mike.  Continuous

transmission (unless squelched) is selected with the -a switch.

1 September 1995

Added ADPCM compression to mike (enabled by the -f switch) and decompression

to speaker.  ADPCM halves the number of bytes in a packet without substantial

degradation of voice-grade audio, far less than that of the -c switch,

which achieves the same degree of compression.  You can specify -c

and -f together for a 4x reduction in bandwidth, with substantial loss

of quality.  ADPCM and GSM compression cannot be used simultaneously.

ADPCM compression is provided primarily as an alternative to Simple

(-c) compression for users with a 64 Kb connection who don't have a

fast enough CPU to run GSM.  ADPCM is much faster on both compression and

decompression than GSM.

2 September 1995

Release 5.1.

15 September 1995

Implemented a crude but better-than-nothing "answering machine" in

speaker.  Specifying the "-rfile" option records all sound received

in the named file in Sun .au format (including header).  If you

specify "-r+file" the sound is appended to the named file if it

exists.  As with a real answering machine with the volume turned up,

the sound is still played.  Obviously there are a zillion nice things

that one can immediately think of which would improve this feature,

but at least it provides a stopgap which captures attempts to

communicate while you were away from the machine.

17 September 1995

Discovered that a "quick fix" in the SGI audio input soundgrab()

function in soundbyte.c was violating the maximum packet size for

memory-constrained versions of WINSOCK. Fixing that was

straightforward, but I ran squarely into a bug where we'd hang trying

to flush queued input when transitioning to talk mode in push-to-talk.

I added a new soundflush() function dedicated to flushing the input

queue (however that is accomplished) and provided Sun and SGI

implementations of this function.

Lots of little changes all over to transition the package name from

Netfone to Speak Freely for Unix.  Much more remains to be done,

particularly separating the option sections for sfmike and sfspeaker

in the man page.